This project is meant for microgrid EMS containing BESS, PV, Genset, Relay and meter assets

- Current state of the project is that I have added modbus servers for read and write for all the assets in PS

- PL logic is implemented for BESS P and PV P command (PID for each in Load Following verilog module) and has been tested data read from PV, BESS and POI meter serve and issue of P command incl. both charge and discharge command

- For remaining assets (Genset, Relay, BESS//PV/Genset/Load meters) I have developed the read interface (to read data from PS to PL) in Verilog. Write (commands from PL to PS) for genset and relay logic has not been included yet

- Next steps should be to implement various control modes/use cases in Verilog (peak shaving, islanding etc)

- In PS, next steps would be to include modbus server (where EMS acts as modbus server and an external controller is able to send commands/setpoints to EMS modbus server), restful api, California CSIP (IEEE 2030.5 via EPRI open source stack), OPC UA and mature webpage based HMI to include all assets